- Anurag Surve

Weekly Report – Week 2

Understanding EEG Data Using the CHB-MIT Scalp EEG Dataset

Week 2 Objective

The main goal of Week 2 was to understand EEG data at a fundamental level before doing any modeling or machine learning.
Instead of directly training models, the focus was on learning:

- What EEG data represents

- How EEG recordings are structured in files

- How seizures are annotated

- How normal EEG and seizure EEG differ visually and statistically

To achieve this, I selected the CHB-MIT Scalp EEG Dataset from PhysioNet.

Dataset Chosen: CHB-MIT Scalp EEG Dataset

The CHB-MIT dataset contains scalp EEG recordings collected from pediatric patients with epilepsy at Children’s Hospital Boston.

Key characteristics of the dataset:

- EEG recordings from 22 patients

- Each patient has multiple EEG recording files

- Seizure start and end times are manually annotated

- Data is commonly used in seizure detection research

Due to local storage limitations, only the first three patient folders were downloaded:

- chb01

- chb02

- chb03

This subset is sufficient for understanding EEG structure and seizure characteristics.

What Files Are Present in Each Patient Folder

Each patient folder contains the following types of files:

1. EDF Files (.edf)

Examples:

- chb01_01.edf

- chb01_02.edf

- chb01_03.edf

These are raw EEG recordings.

Each EDF file contains:

- Continuous EEG signals

- Multiple channels recorded simultaneously

- Usually 1 hour of data per file

2. Summary File (chb01-summary.txt)

This file contains:

- Channel names (electrode pairs)

- Seizure start and end times (in seconds)

- File-level metadata

This file is essential for knowing where seizures occur inside the EEG recordings.

What “Raw Multichannel EEG” Means

- EEG measures electrical activity of the brain.

- Raw means the signal is unfiltered and unprocessed.

- Multichannel means the brain activity is recorded from many locations at the same time.

In this dataset:

- Each file has 23 EEG channels

- Each channel records a separate signal over time

So, one EDF file is essentially:

23 brain signals recorded continuously for ~1 hour

Sampling Rate (256 Hz) – What It Means

The dataset uses a sampling rate of 256 Hz.

This means:

- Each channel records 256 data points per second

- In 1 minute → 256 × 60 = 15,360 samples per channel

- In 1 hour → ~921,600 samples per channel

So EEG data is:

- Very high-resolution

- Very dense

- Time-sensitive

This is why EEG analysis requires careful preprocessing.

Understanding Bipolar Montage Channels

The channels are listed like:

- FP1–F7

- F7–T7

- T7–P7

These are bipolar channels.

What this means:

- Each channel is the difference between two electrodes

- It measures how electrical activity changes between two brain locations

- This helps reduce noise and emphasize localized brain activity

So:

FP1–F7 is NOT one electrode — it is the voltage difference between FP1 and F7.

Understanding Seizure Start and End Times

Example from the summary file:

Seizure Start Time: 2996 seconds

Seizure End Time: 3036 seconds

This means:

- The seizure begins 2996 seconds after the start of that EDF file

- The seizure ends at 3036 seconds

- Duration = 40 seconds

If the file is 1 hour long:

- 2996 seconds ≈ 49 minutes and 56 seconds into the recording

This information allows us to:

- Extract seizure segments

- Compare seizure vs non-seizure EEG activity

Exploratory Data Analysis (EDA) in Python

Python was used to explore the EEG data visually and statistically.

The following analyses were performed:

1. Raw EEG Signal Visualization

Raw EEG signals were plotted for:

- Normal (non-seizure) segments

- Seizure segments

This helped visualize:

- Signal amplitude

- Noise patterns

- Differences in brain activity

Key observation:

- Seizure EEG shows higher amplitude

- Signals appear more synchronized during seizures

2. Power Spectral Density (PSD) Analysis

PSD shows how signal power is distributed across frequencies.

This analysis helps understand:

- Which frequency bands dominate

- How seizure activity differs spectrally

Key observation:

- Seizure segments show higher overall power

- Increased energy in certain frequency bands compared to normal EEG

3. Time–Frequency Analysis (Spectrograms)

Spectrograms show:

- How frequency content changes over time

This is important because:

- Seizures are dynamic events

- Frequency patterns evolve during seizure onset and progression

Key observation:

- Seizure periods show stronger and denser frequency activity

- Clear temporal patterns emerge during seizures

Key Learnings from Week 2

- EEG data is high-dimensional and time-dependent

- Each patient has unique EEG patterns

- Seizure EEG is visually and spectrally different from normal EEG

- Understanding the raw data is critical before applying ML or DL models

Conclusion

Week 2 was focused entirely on understanding the EEG data itself, not modeling.

By exploring:

- File structure

- Sampling rate

- Channel configuration

- Seizure annotations

- Visual and spectral characteristics

A strong foundation was established for future weeks, where preprocessing, feature extraction, and modeling can be performed more meaningfully.