EEG Feature Engineering — Data Dictionary

AdvancedFeatureExtractor (window-level, per-channel) • Generated 2026-02-27

Scope and Output Schema

This document describes every engineered feature produced by AdvancedFeatureExtractor. Features are computed per EEG channel for each window, and optionally include window-level connectivity summaries. Feature names follow the pattern:
  ch<index>_<feature_name>
Example: ch0_mean, ch3_beta_rel, ch12_wav_entropy.

Input window:
  data: numpy array of shape (n_channels, n_times)
  sfreq: sampling frequency (Hz)

Unless otherwise noted, features are unitless or in the natural units implied by the EEG amplitude (e.g., microvolts). When recordings are scaled differently, values scale accordingly; relative features like relative bandpower are scale-invariant.

Reference Papers (Project Sources)

| ID | Title / Venue | Link |
| --- | --- | --- |
| Paper A | EEG Signal Processing for Medical Diagnosis, Healthcare and Monitoring — A Comprehensive Review (IEEE) | https://ieeexplore.ieee.org/document/10353995 |
| Paper B | Frontiers in Artificial Intelligence (2022) — EEG Feature Extraction Review | https://www.frontiersin.org/articles/10.3389/frai.2022.1072801 |
| Paper C | EEG Signal Processing and Feature Extraction — IJMTST (ResearchGate) | https://www.researchgate.net/publication/374337940_EEG_Signal_Processing_and_Feature_Extraction |

Note: Some feature families also have widely-cited original sources (e.g., Hjorth parameters, Sample Entropy). This dictionary cites the above project papers as requested, and mentions original sources when helpful.

Configuration Parameters Affecting Feature Values

| Config Key | Meaning | Default in your fe.yaml | Impact on Features |
| --- | --- | --- | --- |
| fe.sfreq_mode / fe.target_sfreq | Use native EDF sampling or resample to a target Hz | auto / 256 | Changes frequency resolution, PSD/FFT bins, wavelet level feasibility |
| fe.frequency.welch.nperseg_sec | Welch segment length in seconds | 2 | Controls PSD smoothing and variance |
| fe.frequency.welch.fmin / fmax | Frequency range retained for PSD | 0.5 / 50 | Defines total_power, bandpower integration limits |
| fe.frequency.bands | Band edges for delta/theta/alpha/beta/gamma | 0.5–4, 4–8, 8–13, 13–30, 30–50 | Defines band power and relative power |
| fe.frequency.fft.* | FFT dominant frequency settings | fmin=0.5, fmax=50, detrend=True, window=True | Defines FFT search range and pre-processing |
| fe.nonlinear.sampen_m / sampen_r | Sample entropy embedding dimension and tolerance | m=2, r=0.2*std | Strongly affects SampEn magnitude |
| fe.nonlinear.perm_order / perm_delay | Permutation entropy order and delay | order=3, delay=1 | Affects pattern counts and entropy |
| fe.nonlinear.lz_bins | Bins for LZ discretization | 10 | Changes symbolic sequence and complexity |
| fe.wavelet.wavelet / level | Wavelet family and decomposition level | db4 / 4 | Controls coefficients and energy distribution |

Feature Data Dictionary

For each feature: we list the output key, definition, algorithm steps, and interpretation notes.

Note- Signal is Voltage over time and since voltage is calculated by difference between voltages of 2 electrodes. Hence voltage vis-à-vis signal is positive as well as negative

Mean  (Time-domain (per-channel))

| Field | Details |
| --- | --- |
| Output key(s) | ch{c}_mean |
| Why it helps | Captures baseline shift within a window. If referencing is imperfect or artifacts introduce drift, mean can change. |
| Definition / Formula | mean = (1/N) * Σ_{i=1..N} x[i] |
| Algorithm (how computed) | 1) Take the arithmetic average over all samples in the window. |
| Parameters | None. |
| Interpretation | Near 0 for well-referenced EEG; deviations may indicate drift, offset, or sustained deflection. |
| Common pitfalls | Sensitive to DC offset and slow drift; consider high-pass filtering upstream if mean is not meaningful. |
| Project source(s) | Paper A |

Standard Deviation (Std)  (Time-domain (per-channel))

| Field | Details |
| --- | --- |
| Output key(s) | ch{c}_std |
| Why it helps | Measures amplitude variability. Seizure windows often show higher variance due to rhythmic spikes and bursts. |
| Definition / Formula | std = sqrt( (1/N) * Σ (x[i] - mean)^2 ) |
| Algorithm (how computed) | 1) Compute mean. 2) Compute average squared deviation from mean. 3) Take square root. |
| Parameters | Uses population std in numpy (ddof=0). |
| Interpretation | Higher std means larger fluctuations; compare across windows of equal preprocessing and scaling. |
| Common pitfalls | Strongly affected by artifacts (movement, electrode pops). Standardize preprocessing to reduce false positives. |
| Project source(s) | Paper A |

Root Mean Square (RMS)  (Time-domain (per-channel))

| Field | Details |
| --- | --- |
| Output key(s) | ch{c}_rms |
| Why it helps | A stable measure of signal magnitude (energy). Often increases during seizures due to higher amplitude activity. |
| Definition / Formula | rms = sqrt( (1/N) * Σ x[i]^2 ) |
| Algorithm (how computed) | 1) Square samples. 2) Average. 3) Take square root. |
| Parameters | None. |
| Interpretation | RMS is similar to std when mean≈0; RMS remains meaningful even if mean is not exactly zero. |
| Common pitfalls | Artifact-sensitive; amplitude scaling differences across datasets change RMS absolute values. |
| Project source(s) | Paper A |

Skewness  (Time-domain (per-channel))

| Field | Details |
| --- | --- |
| Output key(s) | ch{c}_skew |
| Why it helps | Detects asymmetry in amplitude distribution (e.g., sharp spikes in one direction). |
| Definition / Formula | skew = E[(x - μ)^3] / σ^3 |
| Algorithm (how computed) | Computed using scipy.stats.skew(x) on window samples. |
| Parameters | scipy defaults (bias correction depends on SciPy version; commonly bias=True). |
| Interpretation | Positive skew: more large positive deflections; negative skew: more large negative deflections. |
| Common pitfalls | Unstable for very short windows; highly sensitive to a few extreme samples. |
| Project source(s) | Paper A |

Kurtosis  (Time-domain (per-channel))

| Field | Details |
| --- | --- |
| Output key(s) | ch{c}_kurtosis |
| Why it helps | Measures tail heaviness/peakedness; seizures and spike trains create heavy tails and sharp peaks. |
| Definition / Formula | kurt = E[(x - μ)^4] / σ^4 (implementation may be 'Fisher' or 'Pearson' depending on library) |
| Algorithm (how computed) | Computed using scipy.stats.kurtosis(x). |
| Parameters | scipy defaults (fisher=True by default in many versions: returns excess kurtosis). |
| Interpretation | Higher kurtosis indicates more outliers/spikes relative to a Gaussian-like baseline. |
| Common pitfalls | Confirm whether returned kurtosis is excess (minus 3). Mixing definitions can confuse interpretation. |
| Project source(s) | Paper A |

Line Length  (Time-domain (per-channel))

| Field | Details |
| --- | --- |
| Output key(s) | ch{c}_line_length |
| Why it helps | A classic seizure feature: captures how much the waveform 'moves' within a window. Spikes and fast rhythms increase it. |
| Definition / Formula | LL = Σ_{i=2..N} ¦x[i] - x[i-1]¦ |
| Algorithm (how computed) | 1) Take first difference diff(x). 2) Take absolute values. 3) Sum. |
| Parameters | None. |
| Interpretation | Higher LL = more high-frequency / high-activity waveform within the window. |
| Common pitfalls | Dependent on sampling rate: more samples per second can increase LL for same underlying signal unless resampled consistently. |
| Project source(s) | Paper C |

Zero Crossing Rate (ZCR)  (Time-domain (per-channel))

| Field | Details |
| --- | --- |
| Output key(s) | ch{c}_zcr |
| Why it helps | Counts how often the signal changes sign. Provides a rough oscillation measure without needing FFT. |
| Definition / Formula | ZCR = (1/(N-1)) * Σ I( signbit(x[i]) != signbit(x[i-1]) ) |
| Algorithm (how computed) | 1) Convert samples to sign bits. 2) Count sign changes between adjacent samples. 3) Normalize by number of transitions. |
| Parameters | Uses numpy.signbit and counts changes; effectively includes crossings around 0. |
| Interpretation | Higher ZCR usually indicates faster oscillations or higher noise. |
| Common pitfalls | Sensitive to baseline offset; if mean is not near 0, ZCR may drop artificially. Consider detrending/centering upstream. |
| Project source(s) | Paper A, Paper C |

Hjorth Activity  (Hjorth (per-channel))

| Field | Details |
| --- | --- |
| Output key(s) | ch{c}_hjorth_activity |
| Why it helps | Measures signal power (variance). Seizure bursts typically increase variance. |
| Definition / Formula | Activity = Var(x) |
| Algorithm (how computed) | Compute variance of the window samples. |
| Parameters | Variance computed with numpy.var (ddof=0). |
| Interpretation | Higher activity indicates stronger overall amplitude variation. |
| Common pitfalls | Artifact-sensitive; scaling changes absolute values. |
| Project source(s) | Paper A, Paper C |

Hjorth Mobility  (Hjorth (per-channel))

| Field | Details |
| --- | --- |
| Output key(s) | ch{c}_hjorth_mobility |
| Why it helps | A frequency proxy computed in time-domain. Increases when the signal changes faster (more high-frequency content). |
| Definition / Formula | Mobility = sqrt( Var(dx) / Var(x) ) where dx = diff(x) |
| Algorithm (how computed) | 1) Compute activity = Var(x). 2) Compute dx = diff(x), var_dx = Var(dx). 3) Mobility = sqrt(var_dx / activity). |
| Parameters | Numerical stabilizer: divide by (activity + 1e-12). |
| Interpretation | Higher mobility implies higher mean frequency / faster dynamics. |
| Common pitfalls | Depends on sampling rate and filtering. Compare only with consistent preprocessing and sfreq. |
| Project source(s) | Paper A, Paper C |

Hjorth Complexity  (Hjorth (per-channel))

| Field | Details |
| --- | --- |
| Output key(s) | ch{c}_hjorth_complexity |
| Why it helps | Measures how much the waveform shape deviates from a simple sinusoid. Seizures can raise complexity via irregular morphology. |
| Definition / Formula | Complexity = Mobility(dx) / Mobility(x) where Mobility(dx) = sqrt( Var(ddx) / Var(dx) ), ddx = diff(diff(x)) |
| Algorithm (how computed) | 1) Compute dx and ddx. 2) Compute var_dx and var_ddx. 3) mobility_dx = sqrt(var_ddx/var_dx). 4) complexity = mobility_dx/mobility. |
| Parameters | Stabilizers: add 1e-12 in denominators. |
| Interpretation | ~1 suggests near-sinusoidal behavior; higher values suggest more complex waveform dynamics. |
| Common pitfalls | Noisy derivative estimates in very short windows; consistent window length is important. |
| Project source(s) | Paper A, Paper C |

Sample Entropy (SampEn)  (Nonlinear (per-channel))

| Field | Details |
| --- | --- |
| Output key(s) | ch{c}_sampen |
| Why it helps | Quantifies unpredictability/irregularity of the time series. Seizure transitions can change regularity patterns. |
| Definition / Formula | SampEn(m, r) = -ln( A / B ) B: probability that two sequences match for length m within tolerance r A: probability that they also match for length m+1 Tolerance r is set as r * std(x) in this implementation. |
| Algorithm (how computed) | 1) Compute std(x); tol = r * std(x). 2) Build all length-m embedding vectors. 3) For each vector, count other vectors whose max absolute distance <= tol (Chebyshev distance). 4) Repeat for length m+1. 5) SampEn = -log(A/B). Returns 0 if counts are too small. |
| Parameters | m = fe.nonlinear.sampen_m (default 2), r = fe.nonlinear.sampen_r (default 0.2). |
| Interpretation | Higher SampEn = less predictable (more irregular). Lower SampEn = more regular/repetitive patterns. |
| Common pitfalls | Computationally expensive O(N^2). Sensitive to window length and r; too small r can yield zero matches. |
| Project source(s) | Paper A |

Permutation Entropy (PE)  (Nonlinear (per-channel))

| Field | Details |
| --- | --- |
| Output key(s) | ch{c}_perm_entropy |
| Why it helps | Measures complexity of the ordering (rank patterns) rather than raw amplitude. More robust to monotonic scaling. |
| Definition / Formula | 1) Form ordinal patterns of length 'order' with delay 'delay'. 2) Let p_k be the probability of each ordinal pattern. PE = - Σ p_k log(p_k) (unnormalized in this implementation). |
| Algorithm (how computed) | 1) For each i, take window = x[i : i + delay*order : delay]. 2) Compute rank ordering argsort(window) -> pattern key. 3) Count occurrences of each pattern, normalize to probabilities. 4) Compute Shannon entropy of the pattern distribution. |
| Parameters | order = fe.nonlinear.perm_order (default 3), delay = fe.nonlinear.perm_delay (default 1). |
| Interpretation | Higher PE = more diverse temporal orderings; lower PE = more regular/repetitive ordering. |
| Common pitfalls | Very small windows give few patterns; PE becomes unstable. Normalization (divide by log(factorial(order))) is not applied here. |
| Project source(s) | Paper A |

Lempel-Ziv Complexity (LZ76)  (Nonlinear (per-channel))

| Field | Details |
| --- | --- |
| Output key(s) | ch{c}_lz_complexity |
| Why it helps | Measures how compressible the signal is after discretization. Less compressible sequences yield higher complexity. |
| Definition / Formula | LZ complexity is the count of new distinct substrings encountered when scanning the symbol sequence (LZ76 parsing). |
| Algorithm (how computed) | 1) Discretize x into n_bins symbols using quantile bin edges. 2) Scan the resulting symbol sequence left-to-right. 3) Increment complexity count whenever a new substring not seen before is encountered. 4) Return c (unnormalized). |
| Parameters | n_bins = fe.nonlinear.lz_bins (default 10). |
| Interpretation | Higher values suggest more novelty/irregularity in symbolic dynamics. |
| Common pitfalls | Depends heavily on discretization. Different n_bins changes the symbol sequence and the complexity magnitude. |
| Project source(s) | Paper A |

Welch Total Power  (Frequency-domain (per-channel, Welch PSD))

| Field | Details |
| --- | --- |
| Output key(s) | ch{c}_total_power |
| Why it helps | Measures total spectral energy in [fmin, fmax]. Seizures can increase power in certain ranges and overall power. |
| Definition / Formula | TotalPower = ∫_{fmin..fmax} PSD(f) df (numerical trapezoid integration) |
| Algorithm (how computed) | 1) Compute Welch PSD (scipy.signal.welch) with nperseg. 2) Keep frequencies in [fmin, fmax]. 3) Integrate PSD over frequency using trapezoid rule. |
| Parameters | Welch: nperseg = nperseg_sec * sfreq (default 2 sec). Range: fmin=0.5, fmax=50. |
| Interpretation | Higher total power implies stronger overall oscillatory activity within the analyzed band. |
| Common pitfalls | Absolute value depends on scaling (microvolts vs volts) and preprocessing. Ensure consistent scaling across files. |
| Project source(s) | Paper B |

Band Power (delta/theta/alpha/beta/gamma)  (Frequency-domain (per-channel, Welch PSD))

| Field | Details |
| --- | --- |
| Output key(s) | ch{c}_{band}_power where band ∈ {delta, theta, alpha, beta, gamma} |
| Why it helps | Captures how power is distributed across physiologically meaningful EEG bands. Seizures often redistribute energy. |
| Definition / Formula | BandPower(band) = ∫_{f_lo..f_hi} PSD(f) df |
| Algorithm (how computed) | 1) Compute PSD (Welch). 2) For each band, integrate PSD over the band edges using trapezoid rule. |
| Parameters | Band edges from fe.frequency.bands (default: delta 0.5–4, theta 4–8, alpha 8–13, beta 13–30, gamma 30–50). |
| Interpretation | Higher band power in a band indicates more energy concentrated in that frequency range. |
| Common pitfalls | Band edges must align with dataset sampling and filtering. If fmax < gamma upper bound, gamma_power may be reduced. |
| Project source(s) | Paper B, Paper C |

Relative Band Power  (Frequency-domain (per-channel, Welch PSD))

| Field | Details |
| --- | --- |
| Output key(s) | ch{c}_{band}_rel |
| Why it helps | Normalizes band power by total power, making it more comparable across windows and recordings with different amplitude scaling. |
| Definition / Formula | RelPower(band) = BandPower(band) / (TotalPower + ε) |
| Algorithm (how computed) | 1) Compute total_power in [fmin,fmax]. 2) Compute each band_power. 3) Divide band_power by total_power. |
| Parameters | Enabled by fe.frequency.relative_power (default True). ε = 1e-12 for stability. |
| Interpretation | Values sum to approximately 1 across bands (within numerical and range limitations). |
| Common pitfalls | If total_power is near 0, ratios can be unstable; stabilizer helps but interpret carefully. |
| Project source(s) | Paper C |

Spectral Entropy (PSD-based)  (Frequency-domain (per-channel))

| Field | Details |
| --- | --- |
| Output key(s) | ch{c}_spec_entropy |
| Why it helps | Measures how concentrated vs spread the spectrum is. Rhythmic seizure discharges can concentrate power (lower entropy). |
| Definition / Formula | p_i = PSD_i / Σ PSD_i ; SpectralEntropy = - Σ p_i log(p_i) |
| Algorithm (how computed) | 1) Normalize PSD over kept frequency bins to a probability distribution. 2) Compute Shannon entropy. |
| Parameters | Enabled by fe.frequency.spectral_entropy (default True). Uses natural log. |
| Interpretation | Lower entropy: spectrum dominated by a few frequencies. Higher entropy: flatter, more spread spectrum. |
| Common pitfalls | Entropy depends on the chosen frequency range and PSD resolution (nperseg). Compare only under same settings. |
| Project source(s) | Paper C |

FFT Dominant Frequency  (Frequency-domain (per-channel, FFT))

| Field | Details |
| --- | --- |
| Output key(s) | ch{c}_fft_dom_freq |
| Why it helps | Identifies the strongest oscillatory component within a frequency range. Useful for rhythmic seizure patterns. |
| Definition / Formula | DominantFreq = argmax_{f in [fmin,fmax]} ¦FFT(x)(f)¦ |
| Algorithm (how computed) | 1) Optionally detrend: x <- x - mean(x). 2) Optionally apply Hamming window. 3) Compute rFFT magnitude spectrum. 4) Search only frequencies within [fmin,fmax] and take argmax. 5) Return the corresponding frequency in Hz. |
| Parameters | fmin/fmax from fe.frequency.fft, detrend/window booleans (default True). |
| Interpretation | Returns a single frequency value (Hz). Higher values indicate faster dominant rhythm. |
| Common pitfalls | Dominant frequency is quantized to FFT bin resolution (sfreq/N). Short windows give coarse resolution. |
| Project source(s) | Paper A, Paper B |

Wavelet Approximation Energy  (Time-frequency (per-channel, DWT))

| Field | Details |
| --- | --- |
| Output key(s) | ch{c}_wav_E_A |
| Why it helps | Captures low-frequency energy after wavelet decomposition. Seizures can alter energy at multiple scales. |
| Definition / Formula | E_A = Σ (cA_level)^2 |
| Algorithm (how computed) | 1) Perform DWT decomposition with pywt.wavedec. 2) Compute sum of squares of approximation coefficients. |
| Parameters | Wavelet family (default db4), level (default 4). |
| Interpretation | Higher E_A indicates stronger low-frequency component at the approximation scale. |
| Common pitfalls | Max feasible level depends on window length and wavelet filter length. Implementation clamps to max_level. |
| Project source(s) | Paper A, Paper C |

Wavelet Detail Energy per Level  (Time-frequency (per-channel, DWT))

| Field | Details |
| --- | --- |
| Output key(s) | ch{c}_wav_E_D1 ... ch{c}_wav_E_D{L} |
| Why it helps | Captures energy at progressively finer scales. Transient spikes and bursts often appear in detail coefficients. |
| Definition / Formula | E_Dk = Σ (cD_k)^2 for each detail level k |
| Algorithm (how computed) | 1) DWT decomposition. 2) For each detail coefficient array, compute sum of squares. |
| Parameters | Number of detail energies returned depends on chosen level L (default 4). |
| Interpretation | Higher detail energy at a level suggests stronger activity at that scale. |
| Common pitfalls | Level indexing in the code: energies[1] corresponds to the highest-level detail (cD_level). The naming wav_E_Di follows the code’s index order. |
| Project source(s) | Paper A, Paper C |

Wavelet Entropy  (Time-frequency (per-channel, DWT))

| Field | Details |
| --- | --- |
| Output key(s) | ch{c}_wav_entropy |
| Why it helps | Measures how wavelet energy is distributed across approximation and detail levels. Concentrated energy → lower entropy. |
| Definition / Formula | p_j = E_j / Σ E_j ; WaveletEntropy = - Σ p_j log(p_j) |
| Algorithm (how computed) | 1) Compute energies for [cA, cD_level, ..., cD1]. 2) Normalize energies to probabilities. 3) Compute entropy. |
| Parameters | Enabled by fe.wavelet.entropy (default True). Uses natural log. |
| Interpretation | Lower entropy indicates energy concentrated in fewer scales; higher entropy indicates more spread across scales. |
| Common pitfalls | Depends on chosen wavelet and level; comparing across runs requires identical settings. |
| Project source(s) | Paper C |

Connectivity: Pearson Correlation Summary  (Connectivity (window-level))

| Field | Details |
| --- | --- |
| Output key(s) | conn_corr_mean, conn_corr_std, conn_corr_max, conn_corr_min |
| Why it helps | Summarizes inter-channel synchrony. Seizures often show increased synchronization across electrodes. |
| Definition / Formula | Compute correlation matrix C across channels. Use upper-triangular entries (excluding diagonal): vals. Features: mean(vals), std(vals), max(vals), min(vals). |
| Algorithm (how computed) | 1) Compute C = corrcoef(X) where X has shape (n_channels, n_times). 2) Extract upper triangle (k=1) values. 3) Remove non-finite entries. 4) Compute summary statistics. |
| Parameters | Enabled by fe.connectivity.enabled (default True). Method currently uses Pearson correlation. |
| Interpretation | Higher mean/max correlation indicates stronger similarity among channels within the window. |
| Common pitfalls | Correlation is sensitive to common artifacts and referencing. Bad channels can distort correlation summaries. |
| Project source(s) | Paper B |

Feature Count (Default fe.yaml)

Per-channel features with your default configuration:
  Time-domain: 10
  Nonlinear: 3
  Frequency-domain: 13
  Wavelet: 6
  Total per channel: 32

Connectivity (window-level): 4 features.
Total model features per window: (C * 32) + 4, where C = number of channels.